REVISED DABNEY PROTOCOL CHECKLIST

UIC Williams Ancient DNA Lab 2023-2025

DAY 1 – DECONTAMINATION

[ _ ] UV at least three 50 mL tubes and one sterile bag per sample for 15 min.

[ _ ] Use decontaminated drill or hammer to sample.

[ _ ] Submerge sample in 50 mL tube with 10% bleach, place on rocker for no more than 4 min.

[ _ ] Rinse samples with UltraPure water and place on rocker for 10 min.

[ _ ] Repeat the rinse until the water is clear.

[ _ ] Set samples on sterile weigh boats to dry above the incubator in the hood for at least three day to a week.

DAY 2 – SAMPLE PREP

[ _ ] UV at least two weigh boats, two sterile bags, and one 15 mL tube per sample for 15 min.

[ _ ] UV one mortar and pestle per sample (no more than 5 at time).

[ _ ] Crush sample to a fine powder inside the two sterile bags, transfer to weigh boat

[ _ ] Transfer powdered sample to 15 mL tube

[ _ ] Sterilize all surfaces and equipment before continuing to the next sample

DAY 3 – DIGESTION, SILICA and BUFFER PREP

DIGESTION

[ _ ] UV one 1.5 mL Eppendorf tube per sample and 1 tube for blank extraction control for 15 min.

[ _ ] Add ~120 mg bone/tooth sample to the Eppendorf tube.

[ _ ] Add 1,000 µL 0.45 M EDTA to each tube. Add 12.5 - 15 µl of 20mg/mL Proteinase K to each tube for ~1.1 mL total volume.

[ _ ] Cover tubes in Parafilm, vortex to resuspend bone powder in solution, and incubate on thermal rocker overnight (~18 hours) at 37°C.

Prepare 0.45M EDTA (if needed):

[ _ ] UV one 50mL tube for 15 min.

[ _ ] Add 45 mL 0.5M EDTA and 5 mL water, invert to mix, and UV again for 15 min.

Prepare Silica Suspension (if needed):

[ _ ] UV-irradiate two 50mL tubes, and six 1.5mL tubes for 15 minutes.

[ _ ] Add 4.8g of silicon dioxide to one of the 50 mL tubes.

[ _ ] Add water to a final volume of 40mL, vortex, and let settle for one hour.

[ _ ] Pipetting from the top of the liquid, transfer 39mL of the supernatant to the other 50 mL tube and allow to sediment for an additional four hours.

[ _ ] Pipetting from the top of the liquid, remove and discard 35mL of the supernatant and add 48uL of 30% w/v HCl to the pellet.

[ _ ] Vortex Silica Suspension and aliquot 650uL into each 1.5mL tube.

[ _ ] Store in the dark at room temperature for no longer than one month.

Notes:

- Each tube is enough to decontaminate binding buffer.

- The authors have stated that this solution is too acidic to preserve DNA and thus does not need further treatment for possible DNA contamination.

Prepare Binding Buffer (prepare fresh each time) – Each 50 mL tube contains ~35 mL buffer

[ _ ] Each destructive sample requires 13 mL binding buffer.  ~3 tubes required for set of 5 samples and 1 negative control

[ _ ] UV the necessary number of 50mL tubes, 1 funnel (or weigh boats), and 2 spoons for 15 min.

[ _ ] Add 15mL water and 10mL isopropanol to each tube and UV again for 15 min.

  - If using aliquots in 50 mL tubes, UV those with the cap OFF and add both of these liquids after the powders in the next step:

[ _ ] Add 11.75g guanidine hydrochloride, 0.185 g sodium acetate, (add aliquots of water and isopropanol here) and 12.5 µL Tween-20 to each tube and mix by inverting.

[ _ ] Re-suspend silica suspension by vortexing, and add 100 uL of silica to each tube of binding buffer, wrap with parafilm, and rock overnight at room temperature.

Prepare TET buffer (if needed, only for long-term storage samples):

[ _ ] UV 1 50 mL tube for 15 min.

[ _ ] Add 100 uL 0.5 M EDTA, 500 uL Tris-Cl, and 25 uL Tween 20

[ _ ] Fill to 50 mL with water

[ _ ] Mix by inverting and UV for 15 minutes.

DAY 3 – EXTRACTION

[ _ ] UV one 2mL collection tubes, one 1.5mL Eppendorf capped tube, and one Zymo Extender Assembly for each sample + negative control (6 sets) for 15 min.

[ _ ] UV PE and EB (or TET) buffers for 15 minutes.

  - If necessary, UV more tubes (15 mL for PE and 1.5 mL for TET/EB) to make aliquots of main buffer stocks.

[ _ ] Centrifuge 50 mL tubes containing binding buffer for 2 minutes at 4000 RPM to pellet the silica beads. (~6 ¾ in big centrifuge) (This is time-sensitive, make sure to have everything UVed and set up before continuing to this step)

[ _ ] Remove sample digestion tubes from thermal rocker and centrifuge for 2 minutes at maximum speed (10) to pellet bone powder/debris.

[ _ ] Pipetting from the top of the liquid, transfer 13 mL binding buffer into each Extender assembly, avoiding silica smear/pellet.

[ _ ] Pipette 1 mL (destructive) or 3 mL (non-destructive) of sample supernatant into corresponding Extender Assemblies containing binding buffer, avoiding bone pellet.

[ _ ] Centrifuge Assembly tubes for 4 min at 2500 rpm (Zymo tubes) (~5 ¼ in big centrifuge)

  - Rotate 90° and centrifuge for an additional 2 min if any liquid remains in reservoir. After second spin, check to verify that all of the liquid has passed through the reservoir and spin column. If not, centrifuge for additional 2 minutes.

[ _ ] Remove and discard the Extender and tube containing flow through, and place reservoirs back into assembly.

[ _ ] Dry-spin the spin columns for 1 minute at 3500 rpm (~6 ¼ in big centrifuge).

[ _ ] Add 750 μL of PE buffer (Qiagen) to the spin column (but not directly on filter), centrifuge at 3500 rpm (~6 ¼ in big centrifuge) for 1 minute, and discard the flow-through. Repeat once.

[ _ ] Dry-spin the extended column at 3500 rpm for 1 minute then remove the spin column from the Extender Assembly and place filter into a 2mL collection tube

[ _ ] Dry-spin the column for 2 minutes at maximum speed (10) and place filter column in a 1.5 mL Eppendorf capped tube (Repeat dry-spin if needed for an additional minute)

[ _ ] Pipette 30 μL of elution buffer (EB or TET) directly onto the silica membrane, and incubate at 37 °C for 10 minutes.

[ _ ] Centrifuge for 1 minute at maximum speed (10).

[ _ ] Repeat previous 2 steps for a total of 60 μL of DNA extract.

[ _ ] Discard filter column and store extract at – 20 °C

Note: For samples where library/NGS prep or long term storage is desired, elute in TET (fewer tube effects). For samples where standard PCR or short term storage is desired, samples may be eluted in Qiagen Buffer EB (less inhibition for PCR).