NEBNext Library Checklist – William’s Lab 2025

Day 1 Initial Setup:

-Turn on Incubator and Thermocycler

-UV the following materials:

- 1.5 mL tubes x3 (one for End Prep MM, one for diluted adaptor, one for Ligation MM)

- PCR tubes x2 per sample (one for End Prep, one after Cleanup)

- 15 mL tube (for fresh 80% ethanol)

- Aliquot of UltraPure DNAse-free water

-Take out magnet for PCR tubes

-Prepare reagents

- NEBNext Ultra II DNA Kit

- NEBNExt Dual Index Primers Box

- Purification beads

- 10mM Tris-HCL (or 0.1x TE)

1. End Prep:

[ _ ] Make End Prep Master Mix (Green) (pipette reagents, do not vortex):

[ _ ] 3 μl End Prep Enzyme Mix per sample

[ _ ] 7 μl End Prep Reaction Buffer per sample

[ _ ] Add 10 μl End Prep Master Mix to PCR tubes

[ _ ] Add 50 μl DNA extract

[ _ ] Pipette mix 10 timers with pipette set to 50 μl, quick spin

[ _ ] Thermal cycle (NEBLIBEP: Heated lid ≥ 75°C; 30 min 20°C, 30 min 65°C, Hold at 4°C)

Meanwhile, make sure the beads are at room temperature for at least 45 minutes, prepare 80% ethanol (320 μl ethanol and 80 μl UltraPure H2O, 400 μl total per sample), and balance the PCR centrifuge according to your number of samples.

2. Adaptor Ligation:

[ _ ] Dilute the adaptor 1:15 (2 μl Adaptor + 30 μl 10mM Tris-HCL)

[ _ ] Make Ligation Master Mix (Red) (pipette reagents, do not vortex):

[ _ ] 30 μl Ligation Master Mix per sample

[ _ ] 1 μl Ligation Enhancer per sample

[ _ ] Add 31 μl Ligation Master Mix to each sample

[ _ ] Add 2.5 μl diluted adaptor to each sample

[ _ ] Pipette mix 10 timers with pipette set to 80 μl, quick spin

[ _ ] Incubate 15 min at 20°C (room temperature)

[ _ ] Add 3 μl USER (found in primer box)

[ _ ] Pipette mix 10 timers with pipette set to 80 μl, quick spin

[ _ ] Incubate 15 minutes at 37°C (incubator)

3. Cleanup of Adaptor-Ligated DNA without Size Selection:

[ _ ] Vortex beads, add 87 μl of room temperature beads to sample and mix by pipetting 10 times

[ _ ] Incubate for 10 min at room temperature

[ _ ] Place tubes on magnet

[ _ ] Wait 5 minutes and remove supernatant (keep the beads, do not disturb)

[ _ ] On magnet add 200 μl fresh 80% ethanol, wait 30 seconds and remove

[ _ ] Repeat previous step once and use long pipette tip to remove all liquid

[ _ ] Air dry beads (3 min, do not overdry!)

[ _ ] Off magnet add 17 μl  10mM Tris-HCL (0.1x TE also works)

[ _ ] Mix by pipetting 10 times, incubate for 2 min at room temperature

[ _ ] Place tubes on magnet, wait 5 min and transfer 15 μl to a new PCR tube

YOU CAN STOP HERE AND STORE SAMPLES AT -20°C OVERNIGHT

Day 2

4. PCR Enrichment of Adaptor-Ligated DNA:

[ _ ] Add PCR Reagents to Samples (Blue):

[ _ ] 5 μl i7 primer, 5 μl i5 primer

[ _ ] 25 μl Q5 Master Mix (pipette to mix, do not vortex)

[ _ ] Pipette mix 10 times with pipette set to 40 μl, quick spin

[ _ ] Take to Modern DNA lab for Thermal Cycle (NEBNEXT-14: Heated lid ≥ 103°C; 98°C 30 sec, 14 cycles of 98°C for 10 sec and 65°C for 75 sec, 65°C for 5 min, Hold at 4°C)

While the Thermocycler is running (~52 min), make sure the beads are at room temperature for at least 45 minutes, aliquot 400 μl of 70% ethanol per sample, and set up new PCR tubes for the final libraries.

5. Cleanup of PCR Reaction:

[ _ ] Vortex beads

[ _ ] Add 45 μl of room temperature beads to sample and mix by pipetting 10 times

[ _ ] Incubate for 10 min

[ _ ] Place tubes on magnet

[ _ ] Wait 5 min and remove supernatant (keep the beads, do not disturb)

[ _ ] On magnet add 200 μl 70% ethanol, wait 30 seconds and remove

[ _ ] Repeat previous step once and use long pipette tip to remove all liquid

[ _ ] Air dry beads (3 min, do not overdry!)

[ _ ] Off magnet add 33 μl  0.1x TE (10mM Tris-HCL also works)

[ _ ] Mix by pipetting 10 times, incubate for 2 min at room temperature

[ _ ] Place tubes on magnet, wait 5 min and transfer 30 μl to a new PCR tube

Store Libraries at -20°C.